ABSTRACT

This paper presents a smart highway navigation system by using Li-Fi technology with audio alert. The light fidelity or li-fi technology refers to visible light communication that uses light as a medium to deliver high-speed data in a manner which is much greater than that of Wi-Fi.

Over here we demonstrate a hardware prototype which is developed using Arduino microcontroller platform to explore the possibilities of using Li-Fi in highway routing. The transmitter and receiver sections contain atmega328 which is programmed by using Arduino IDE.

High intensity LEDs are used in the transmitter section for delivering high-speed data to moving vehicles. Further, in the receiver section LDR module is used to detect the signal generated by the LEDs. According to the received signal the information of present location and further diversions is displayed on the LCD installed at the receiver.

One new feature which we plan to add is audio feedback in which the receiver device will play the location information using a mp3 player and speaker system. This is to provide benefit to the users to understand their location easily. We wish to reduce the dependency on GPS devices and make Li-Fi a commonly used technology for the future.

Thus, this technology is more useful for automatic navigation on highways and broad-lanes.

# Chapter 1 Introduction

1.1 WHAT IS LIFI?

Li-Fi, or "Light fidelity", refers to wireless communication systems using light from light-emitting diodes as a medium instead of traditional radio frequencies, as in technology using the trademark Wi-Fi. Li-Fi is expected to be ten times cheaper than Wi-Fi. Li-Fi has the advantage of being able to be used in electromagnetic sensitive areas such as in aircraft and nuclear power plants without causing interference. The light waves cannot penetrate walls which makes a much shorter range, though more secure from hacking, relative to Wi-Fi. 

While the US Federal Communications Commission has warned of a potential spectrum crisis because Wi-Fi is close to full capacity, Li-Fi has almost no limitations on capacity. The visible light spectrum is 10,000 times larger than the entire radiofrequency spectrum. Researchers have reached data rates of 3.5 Gbps and have set a goal of reaching 6 Gbps. The Li-Fi market is projected to be worth over $6 billion per year by 2018. Low reliability and high installation costs are the potential drawbacks.

The general term visible light communication (VLC), includes any use of the visible light portion of the electromagnetic spectrum to transmit information. The term Li-Fi was coined by Harald Haas from the University of Edinburgh in the UK. The D-Light project at Edinburgh's Institute for Digital Communications was funded from January 2010 to January 2012.Haas promoted this technology in his 2011 TED Global talk and helped start a company to market it. Pure VLC is an original equipment manufacturer (OEM) firm set up to commercialize Li-Fi products for integration with existing LED-lighting systems.

In October 2011, companies and industry groups formed the Li-Fi Consortium, to promote high-speed optical wireless systems and to overcome the limited amount of radio-based wireless spectrum available by exploiting a completely different part of the electromagnetic spectrum. A number of companies offer unidirectional VLC products.

VLC technology was exhibited in 2012 using Li-Fi. By August 2013, data rates of over 1.6 Gbps were demonstrated over a single-color LED. In September 2013, a press release said that Li-Fi, or VLC systems in general, do not require line-of-sight conditions. In October 2013, it was reported Chinese manufacturers were working on Li-Fi development kits.

VLC communication is modeled after communication protocols established by the IEEE 802 workgroup. This standard defines the physical layer (PHY) and media access control (MAC) layer. The standard is able to deliver enough data rates to transmit audio, video and multimedia services. It takes count of the optical transmission mobility, its compatibility with artificial lighting present in infrastructures, the defiance which may be caused by interference generated by the ambient lighting. The MAC layer allows to use the link with the other layers like the TCP/IP protocol.

FIG 1.1 LI-FI CONCEPT

1.2 OBJECTIVE

To create a new class of high intensity light source of solid-state design bringing clean lighting solutions to general and specialty lighting.  With energy efficiency, long useful lifetime, full spectrum and dimming, LI-FI lighting applications work better compared to conventional approaches.  This technology brief describes the general construction of LI-FI lighting systems and the basic technology building blocks behind their function.

1.3 LITERATURE SURVEY

As more and more people and their many devices access wireless internet, clogged airwaves are going to make it increasingly difficult to latch onto a reliable signal. But radio waves are just one part of the spectrum that can carry our data. What if we could use other waves to surf the internet? One German physicist, DR. Harald Haas, has come up with a solution he calls “Data Through Illumination”—taking the fiber out of fiber optics by sending data through an LED light bulb that varies in intensity faster than the human eye can follow. It’s the same idea behind infrared remote controls, but far more powerful.

Haas says his invention, which he calls D-Light, can produce data rates faster than 10 megabits per second, which is speedier than your average broadband connection. He envisions a future where data for laptops, smart phones, and tablets is transmitted through the light in a room. And security would be a snap—if you can’t see the light, you can’t access the data.

Li-Fi is now part of the Visible Light Communications (VLC) PAN IEEE 802.15.7 standard. Li-Fi is typically implemented using white LED light bulbs. These devices are normally used for illumination by applying a constant current through the LED. However, by fast and subtle variations of the current, the optical output can be made to vary at extremely high speeds. Unseen by the human eye, this variation is used to carry high-speed data.

FIG 1.2 DATA STREAMING

1.4 MOTIVATION

The capacity of radio waves is expensive, less bandwidth, insufficient spectrum
 and availability of these radio waves is limited. These radio waves are not available in aircrafts or in hospitals. They are congested due to the high usage of 2G,3G,4G. The base stations used are only 5% efficient .95% of the efficiency is used in cooling the base stations. the security is also suffered. The Wi-Fi is penetrable through the wall. Security is the big issue here. One can hack the password. An unauthorized person can access the private matters. It is less economic. Availability of radio waves is less hence tariff rates are high. Due to this it is highly expensive. Moreover, radio waves cannot pass under sea.

These drawbacks gave rise to find a new invention and a new method of communication that is visible light communication. This technology is yet to come in the market that is based on data communication through light.

1.5 PROPOSED IDEA

It is quite often that traveller travelling to remote areas find it difficult to get onto the right path. The problem grows up during night, due to lack of visibility, non-functioning of GPS since not everywhere on the highway we find the mobile networks which can be used for navigation.

At that time if there is another mean which can provide the traveller with the right pathway and location, it will surely reduce the panic and encourage more travellers to take the road trip.

Thus to tackle with this problem, we propose to install Li-Fi in the streetlights. Li-Fi (light fidelity technology) is the recent technology that emerged in the Field of wireless communication. It is a part of visible light communication (VLC) PAN IEEE 802.15.7 standard. It is transmission of data using visible light by sending data through LED bulbs that switches faster than the human eye can detect. 

Li-Fi is wireless communication which is an improvised technology of Wi-Fi but it is different in every aspect such as data transmission rates, security, high frequency and bandwidth, etc. We are living in an era where we are surrounded by wireless technologies around us like cell phones, Wi-Fi, IOT, etc.

Nowadays, everyone wants most of the things based on wireless technologies. It includes the use of radio waves for communication like Bluetooth & Wi-Fi and also the new emerging technology Li-Fi. The Li-Fi technology uses LEDs for data transmission. Li-Fi is rapidly growing and taking a place in the market as it is fast and secure compared to other wireless technologies and can transmit the data at a very high speeds.

 There are many researches are going in these fields such as: “Li-Fi the path to a new way of communication”. These units have microcontrollers having data previously stored in it (i.e. direction indications). As soon as a vehicle comes in the range of the visible light of these poles, it transmits the data to that vehicle. The available information gets displayed on the LCD installed with receiver in the vehicle. This Li-Fi based highway navigation system where the LEDs that are used in streetlights for illumination purpose will also be providing the travellers with the information of the present location and all the divergence ahead. This project has a very wide scope in near future as it can help setup outdoor as well as indoor navigation system. The main idea of our paper is to create a navigation system for the areas like highways and broad-lanes to create automatic navigation for the travellers using Li-Fi technology. This will revolutionize the highway routing as it become a better alternative to GPS system.

# Chapter 2

# Hardware

2.1 Hardware Components Used:

- Arduino Nano/Uno (Based on ATmega 328P Microcontroller)

- 16 x 2 LCD Display

- LDR Sensor Circuit

- TIP122 Power Transistor

- Power Supply

2.2 MICROCONTROLLER

Microcontroller, as the name suggests, are small controllers. They are like single chip computers that are often embedded into other systems to function as processing/controlling unit. For example, the remote control you are using probably has microcontrollers inside that do decoding and other controlling functions. They are also used in automobiles, washing machines, microwave ovens, toys, etc, where automation is needed.

2.2.1 Description

Arduino is an open-source electronics prototyping platform based on flexible, easy-to-use hardware and software. It's intended for artists, designers, hobbyists, and anyone interested in creating interactive objects or environments. Arduino can sense the environment by receiving input from a variety of sensors and can affect its surroundings by controlling lights, motors, and other actuators. The microcontroller on the board is programmed using the Arduino programming language (based on Wiring) and the Arduino development environment (based on Processing). Arduino projects can be stand-alone or they can communicate with software running on a computer (e.g. Flash, Processing, MaxMSP).

The boards can be built by hand or purchased preassembled; the software can be downloaded for free. The hardware reference designs (CAD files) are available under an open-source license, you are free to adapt them to your needs. Arduino received an Honorary Mention in the Digital Communities section of the 2006 Ars Electronica Prix. The Arduino team is: Massimo Banzi, David Cuartielles, Tom Igoe, Gianluca Martino, and David Mellis.

Fig. 2.1 Arduino Boards

2.2.2 Hardware

An Arduino board consists of an 8-bit Atmel AVR microcontroller with complementary components to facilitate programming and incorporation into other circuits. An important aspect of the Arduino is the standard way that connectors are exposed, allowing the CPU board to be connected to a variety of interchangeable add-on modules known as shields. Some shields communicate with the Arduino board directly over various pins, but many shields are individually addressable via an I²C serial bus, allowing many shields to be stacked and used in parallel. Official Arduinos have used the megaAVR series of chips, specifically the ATmega8, Atmega168, Atmega328, Atmega1280, and Atmega2560.

A handful of other processors have been used by Arduino compatibles. Most boards include a 5 volt linear regulator and a 16 MHz crystal oscillator (or ceramic resonator in some variants), although some designs such as the LilyPad run at 8 MHz and dispense with the onboard voltage regulator due to specific form-factor restrictions. An Arduino’s microcontroller is also pre-programmed with a boot loader that simplifies uploading of programs to the on-chip flash memory, compared with other devices that typically need an external programmer.At a conceptual level, when using the Arduino software stack, all boards are programmed over an RS-232 serial connection, but the way this is implemented varies by hardware version. Serial Arduino boards contain a simple inverter circuit to convert between RS-232-level and TTL-level signals. Current Arduino boards are programmed via USB, implemented using USB-to-serial adapter chips such as the FTDI FT232.

Some variants, such as the Arduino Mini and the unofficial Boarduino, use a detachable USB-to-serial adapter board or cable, Bluetooth or other methods. (When used with traditional microcontroller tools instead of the Arduino IDE, standard AVR ISP programming is used.)The Arduino board exposes most of the microcontroller’s I/O pins for use by other circuits.

The Diecimila, Duemilanove, and current Uno provide 14 digital I/O pins, six of which can produce pulse-width modulated signals, and six analog inputs. These pins are on the top of the board, via female 0.1 inch headers.

Some types of Arduino Boards available:

ARDUINO MICRO

ARDUINO LILY

ARDUINO NANO

ARDUINO PRO

ARDUINO LEONARDO

ARDUINO FIO

2.2.2.1 Hardware of Arduino UNO

The Arduino Uno is a microcontroller board based on the Atmega328 (datasheet). It has 14 digital input/output pins (of which 6 can be used as PWM outputs), 6 analog inputs, a 16 MHz ceramic resonator, a USB connection, a power jack, an ICSP header, and a reset button. It contains everything needed to support the microcontroller; simply connect it to a computer with a USB cable or power it with a AC-to-DC adapter or battery to get started. The Uno differs from all preceding boards in that it does not use the FTDI USB-to-serial driver chip. Instead, it features the Atmega16U2 (Atmega8U2 up to version R2) programmed as a USB-to-serial converter. Revision 2 of the board has the following new features:

- pinout: added SDA and SCL pins that are near to the AREF pin and two other new pins placed near to the RESET pin, the IOREF that allow the shields to adapt to the voltage provided from the board. In future, shields will be compatible both with the board that use the AVR, which operate with 5V and with the Arduino Due that operate with 3.3V. The second one is a not connected pin, that is reserved for future purposes.

Stronger RESET circuit.

Atmega 16U2 replace the 8U2.

“Uno” means one in Italian and is named to mark the upcoming release of Arduino 1.0. The Uno and version 1.0 will be the reference versions of Arduino, moving forward. The Uno is the latest in a series of USB Arduino boards, and the reference model for the Arduino platform.

2.2 Arduino Uno (Front and Back)

2.2.2.1.A  Power

The Arduino Uno can be powered via the USB connection or with an external power supply. The power source is selected automatically. External (non-USB) power can come either from an AC-to-DC adapter (wall-wart) or battery. The adapter can be connected by plugging a 2.1mm center-positive plug into the board's power jack. Leads from a battery can be inserted in the Gnd and Vin pin headers of the POWER connector.

The board can operate on an external supply of 6 to 20 volts. If supplied with less than 7V, however, the 5V pin may supply less than five volts and the board may be unstable. If using more than 12V, the voltage regulator may overheat and damage the board. The recommended range is 7 to 12 volts.

The power pins are as follows:

VIN. The input voltage to the Arduino board when it's using an external power source (as opposed to 5 volts from the USB connection or other regulated power source). You can supply voltage through this pin, or, if supplying voltage via the power jack, access it through this pin.

5V.This pin outputs a regulated 5V from the regulator on the board. The board can be supplied with power either from the DC power jack (7 - 12V), the USB connector (5V), or the VIN pin of the board (7-12V). Supplying voltage via the 5V or 3.3V pins bypasses the regulator, and can damage your board. We don't advise it.

3V3. A 3.3 volt supply generated by the on-board regulator. Maximum current draw is 50 mA.

GND. Ground pins.

2.2.2.1.B Memory

The ATmega328 has 32 KB (with 0.5 KB used for the bootloader). It also has 2 KB of SRAM and 1 KB of EEPROM (which can be read and written with the EEPROM library).

2.2.2.1.C Input and Output

Each of the 14 digital pins on the Uno can be used as an input or output, using pinMode(), digitalWrite(), and digitalRead() functions. They operate at 5 volts. Each pin can provide or receive a maximum of 40 mA and has an internal pull-up resistor (disconnected by default) of 20-50 kOhms. In addition, some pins have specialized functions:

Serial: 0 (RX) and 1 (TX). Used to receive (RX) and transmit (TX) TTL serial data. These pins are connected to the corresponding pins of the ATmega8U2 USB-to-TTL Serial chip.

External Interrupts: 2 and 3. These pins can be configured to trigger an interrupt on a low value, a rising or falling edge, or a change in value. See the attachInterrupt() function for details.

PWM: 3, 5, 6, 9, 10, and 11. Provide 8-bit PWM output with the analogWrite() function.

SPI: 10 (SS), 11 (MOSI), 12 (MISO), 13 (SCK). These pins support SPI communication using the SPI library.

LED: 13. There is a built-in LED connected to digital pin 13. When the pin is HIGH value, the LED is on, when the pin is LOW, it's off.

The Uno has 6 analog inputs, labeled A0 through A5, each of which provide 10 bits of resolution (i.e. 1024 different values). By default they measure from ground to 5 volts, though is it possible to change the upper end of their range using the AREF pin and the analogReference() function. Additionally, some pins have specialized functionality:

TWI: A4 or SDA pin and A5 or SCL pin. Support TWI communication using the Wire library.

There are a couple of other pins on the board:

AREF. Reference voltage for the analog inputs. Used with analogReference().

Reset. Bring this line LOW to reset the microcontroller. Typically used to add a reset button to shields which block the one on the board.

2.2.2.1.D Communication

The Arduino Uno has a number of facilities for communicating with a computer, another Arduino, or other microcontrollers. The ATmega328 provides UART TTL (5V) serial communication, which is available on digital pins 0 (RX) and 1 (TX). An ATmega16U2 on the board channels this serial communication over USB and appears as a virtual com port to software on the computer. The '16U2 firmware uses the standard USB COM drivers, and no external driver is needed. However, on Windows, a .inf file is required. The Arduino software includes a serial monitor which allows simple textual data to be sent to and from the Arduino board. The RX and TX LEDs on the board will flash when data is being transmitted via the USB-to-serial chip and USB connection to the computer (but not for serial communication on pins 0 and 1).

2.2.2.1.E Programming

The Arduino Uno can be programmed with the Arduino software. Select "Arduino Uno from the Tools > Board menu (according to the microcontroller on your board)..

The ATmega328 on the Arduino Uno comes preburned with a bootloader that allows you to upload new code to it without the use of an external hardware programmer. It communicates using the original STK500 protocol

2.2.2.1.E  USB Over current Protection

The Arduino Uno has a resettable polyfuse that protects your computer's USB ports from shorts and overcurrent. Although most computers provide their own internal protection, the fuse provides an extra layer of protection. If more than 500 mA is applied to the USB port, the fuse will automatically break the connection until the short or overload is removed.

2.2.2.1.F  Physical Characteristics

The maximum length and width of the Uno PCB are 2.7 and 2.1 inches respectively, with the USB connector and power jack extending beyond the former dimension. Four screw holes allow the board to be attached to a surface or case. Note that the distance between digital pins7 and 8 is 160 mil (0.16"), not an even multiple of the 100 mil spacing of the other pins.

Fig. 2.3 Pin Diagram of the UNO along with mapping of Atmega328 pins

2.2.3 Summary

| Microcontroller | ATmega328 |
| --- | --- |
| Operating Voltage | 5V |
| Input Voltage (recommended) | 7-12V |
| Input Voltage (limits) | 6-20V |
| Digital I/O Pins | 14 (of which 6 provide PWM output) |
| Analog Input Pins | 6 |
| DC Current per I/O Pin | 40 mA |
| DC Current for 3.3V Pin | 50 mA |
| Flash Memory | 32 KB (ATmega328) of which 0.5 KB used by bootloader |
| SRAM | 2 KB (ATmega328) |
| EEPROM | 1 KB (ATmega328) |
| Clock Speed | 16 MHz |

2.2 Block Diagram of AVR ATmega328

Fig. 2.4 Block Diagram

Fig. 2.5 The Arduino Uno Pinout

2.3 LCD (Liquid Crystal Display)

Frequently, a program must interact with the outside world using input and output devices that communicate directly with a human being. One of the most common devices attached to a controller is an LCD display. Some of the most common LCDs connected to the Arduino are 16x2 and 20x2 displays. This means 16 characters per line by 2 lines and 20 characters per line by 2 lines, respectively.

Fortunately, a very popular standard exists which allows us to communicate with the vast majority of LCDs regardless of their manufacturer. The standard is referred to as HD44780U, which refers to the controller chip which receives data from an external source (in this case, the UNO) and communicates directly with the LCD.

2.3.1 44780 Backgroud

The 44780 standard requires 3 control lines as well as either 4 or 8 I/O lines for the data bus. The user may select whether the LCD is to operate with a 4-bit data bus or an 8-bit data bus. If a 4-bit data bus is used the LCD will require a total of 7 data lines (3 control lines plus the 4 lines for the data bus). If an 8-bit data bus is used the LCD will require a total of 11 data lines (3 control lines plus the 8 lines for the data bus).

The three control lines are referred to as EN, RS, and RW.

The EN line is called "Enable." This control line is used to tell the LCD that you are sending it data. To send data to the LCD, your program should make sure this line is low (0) and then set the other two control lines and/or put data on the data bus. When the other lines are completely ready, bring EN high (1) and wait for the minimum amount of time required by the LCD datasheet (this varies from LCD to LCD), and end by bringing it low (0) again.

The RS line is the "Register Select" line. When RS is low (0), the data is to be treated as a command or special instruction (such as clear screen, position cursor, etc.). When RS is high (1), the data being sent is text data which should be displayed on the screen. For example, to display the letter "T" on the screen you would set RS high.

The RW line is the "Read/Write" control line. When RW is low (0), the information on the data bus is being written to the LCD. When RW is high (1), the program is effectively querying (or reading) the LCD. Only one instruction ("Get LCD status") is a read command. All others are write commands--so RW will almost always be low.

Finally, the data bus consists of 4 or 8 lines (depending on the mode of operation selected by the user). In the case of an 8-bit data bus, the lines are referred to as DB0, DB1, DB2, DB3, DB4, DB5, DB6, and DB7.

As we've mentioned, the LCD requires either 8 or 11 I/O lines to communicate with. For the sake of this tutorial, we are going to use an 8-bit data bus--so we'll be using 11 of the 8051's I/O pins to interface with the LCD.

Let's draw a sample pseudo-schematic of how the LCD will be connected to the UNO.

Fig 2.6 LCD-Microcontroller Interfacing

As you can see, we've established a 1-to-1 relation between a pin on the UNO and a line on the 44780 LCD. Thus we write our sketch to access the LCD, by including library LIQUIDCRYSTAL.H and using the in build functions provided by Arduino IDE. Following is a list of functions that can be used for LCD.

2.3.2 List of Functions

- Hello World: displays "hello world!" and the seconds since reset.

- Blink: control of the block-style cursor.

- Cursor: control of the underscore-style cursor.

- Display: quickly blank the display without losing what's on it.

- TextDirection: control which way text flows from the cursor.

- Scroll: scroll text left and right.

- Serial input: accepts serial input, displays it.

- SetCursor: set the cursor position.

- Autoscroll: shift text right and left.

Fig 2.7 LCD Display

2.4 LDR (Light Dependent Resistor) Sensor

A Light Dependent Resistor (aka LDR, photoconductor, photocell, or photoresistor.) is a device which has a resistance which varies according to the amount of light falling on its surface, when light falls upon it then the resistance changes.

Light dependent resistors or LDRs are often used in circuits where it is necessary to detect the presence of light, or the ambient level of light, often to create a light triggered switch.

Different LDR’s have different specifications, a typical LRD has a resistance in total darkness of 1 MOhm, and a resistance of a couple of kOhm in bright light (10-20kOhm @ 10 lux, 2-4kOhm @ 100 lux). It is not uncommon for the values of resistance of an LDR to be several megohms in darkness and then to fall to a few hundred ohms in bright light. 

With such a wide variation in resistance, LDRs are easy to use and there are many LDR circuits available. LDRs are made from semiconductor materials to enable them to have their light sensitive properties. Many materials can be used, but one popular material for these LDR’s is cadmium sulphide (CdS).

Fig 2.8 LDR Sensor Depiction

2.4.1 Uses for Light Dependent Resistors

Light dependent resistors are a vital component in any electric circuit which is to be turned on and off automatically according to the level of ambient light - for example, solar powered garden lights, and night security lighting.

Fig 2.9

2.4.2 Working of LDR

To understand how an LDR works, It is first necessary to understand that an electrical current consists of the movement of electrons within a material. Good conductors have a large number of free electrons that can drift in a given direction under the action of a potential difference. Insulators with a high resistance have very few free electrons, and therefore it is hard to make the them move and hence a current to flow.

An LDR is made of semiconductor material with a high resistance. It has a high resistance because there are very few electrons that are free and able to move - the vast majority of the electrons are locked into the crystal lattice and unable to move. Therefore, in this state there is a high LDR resistance.

As light falls on the semiconductor, the light photons are absorbed by the semiconductor lattice and some of their energy is transferred to the electrons. This gives some of them sufficient energy to break free from the crystal lattice so that they can then conduct electricity. This results in a lowering of the resistance of the semiconductor and hence the overall LDR resistance.

The process is progressive, and as more light shines on the LDR semiconductor, so more electrons are released to conduct electricity and the resistance falls further.

2.5 Transistor

The name is transistor derived from ‘transfer resistors’ indicating a solid state semiconductor device. In addition to conductor and insulators, there is a third class of material that exhibits proportion of both. Under some conditions, it acts as an insulator, and under other conditions it’s a conductor. This phenomenon is called semi-conducting and allows a variable control over electron flow. So, the transistor is semi conductor device used in electronics for amplitude.

Transistor has three terminals, one is the collector, one is the base and other is the emitter, (each lead must be connected in the circuit correctly and only then the transistor will function). Electrons are emitted via one terminal and collected on another terminal, while the third terminal acts as a control element. Each transistor has a number marked on its body. Every number has its own specifications.

There are mainly two types of transistor (i) npn & (ii) pnp

2.5.1 NPN transistors:

When a positive voltage is applied to the base, the transistor begins to conduct by allowing current to flow through the collector to emitter circuit. The relatively small current flowing through the base circuit causes a much greater current to pass through the emitter / collector circuit.  The phenomenon is called current gain and it is measure in beta.

2.5.2 PNP transistor:

It also does exactly same thing as above except that it has a negative voltage on its collector and a positive voltage on its emitter.

Transistor is a combination of semi-conductor elements allowing a controlled current flow. Germanium and silicon is the two semi-conductor elements used for making it. There are two types of transistors such as point contact and junction transistors. Point contact construction is defective so is now out of use. Junction triode transistors are in many respects analogous to triode electron tube.

# 2.5.3 Power transistor

If a transistor designed to these concepts can handle more than 1 A of collector current, it is generally considered to be a power transistor. Such transistors must have low output resistance to deliver large load currents, and good junction insulation to resist high voltages. 

They must also dissipate heat very quickly to avoid overheating. The collector/base junction must be as large as possible, as most heat is generated in this area. For our project, we are using TIP122 so that large LED’s can also be driven by our system.

2.6 LED (Light Emitting Diode)

In the simplest terms, a light-emitting diode (LED) is a semiconductor device that emits light when an electric current is passed through it. Light is produced when the particles that carry the current (known as electrons and holes) combine together within the semiconductor material.

Since light is generated within the solid semiconductor material, LEDs are described as solid-state devices. The term solid-state lighting, which also encompasses organic LEDs (OLEDs), distinguishes this lighting technology from other sources that use heated filaments (incandescent and tungsten halogen lamps) or gas discharge (fluorescent lamps).

2.6.1 Different colors
Inside the semiconductor material of the LED, the electrons and holes are contained within energy bands. The separation of the bands (i.e. the bandgap) determines the energy of the photons (light particles) that are emitted by the LED.

The photon energy determines the wavelength of the emitted light, and hence its color. Different semiconductor materials with different bandgaps produce different colors of light. The precise wavelength (color) can be tuned by altering the composition of the light-emitting, or active, region.

LEDs are comprised of compound semiconductor materials, which are made up of elements from group III and group V of the periodic table (these are known as III-V materials). Examples of III-V materials commonly used to make LEDs are gallium arsenide (GaAs) and gallium phosphide (GaP).

Until the mid-90s LEDs had a limited range of colors, and in particular commercial blue and white LEDs did not exist. The development of LEDs based on the gallium nitride (GaN) material system completed the palette of colors and opened up many new applications.

2.6.2 Main LED materials
The main semiconductor materials used to manufacture LEDs are:

- Indium gallium nitride (InGaN): blue, green and ultraviolet high-brightness LEDs

- Aluminum gallium indium phosphide (AlGaInP): yellow, orange and red high-brightness LEDs

- Aluminum gallium arsenide (AlGaAs): red and infrared LEDs

- Gallium phosphide (GaP): yellow and green LEDs

# Chapter 3

# Block Diagram

MP3 AUDIO PLAYER

SPEAKER

16X2 LCD

ARDUINO CIRCUIT

LIGHT SENSOR

LDR

RECEIVER

POLE 1

POLE 2

POLE 1

TRANSMITTER

LED DRIVER 1

LED DRIVER 2

LED DRIVER 3

AMPLIFER       &

FILTER

ARDUINO CIRCUIT

# Chapter 4

# Software

Connecting hardware according to the circuit diagram doesn’t work really. You need to program the components accordingly so as to work according to the algorithm you have designed in order to make the project work properly as required. Following is the description of software used.

4.1 Arduino IDE

The Arduino IDE is a cross-platform application written in Java, and is derived from the IDE for the Processing programming language and the Wiring project. It is designed to introduce programming to artists and other newcomers unfamiliar with software development. It includes a code editor with features such as syntax highlighting, brace matching, and automatic indentation, and is also capable of compiling and uploading programs to the board with a single click. There is typically no need to edit makefiles or run programs on a command-line interface. Although building on command-line is possible if required with some third-party tools such as Ino. The Arduino IDE comes with a C/C++ library called "Wiring" (from the project of the same name), which makes many common input/output operations much easier.

4.1.1 Key Features

- Cross-platoform program

- Open-Source

- Different instructions for different OS

- Example Programs

4.1.2 Blink Program

Once we have downloaded/unzipped the arduino IDE, we can Plug the Arduino to our PC via USB cable.We are actually ready to “burn” our first program on the arduino board. To select “blink led”, the physical translation of the well known programming “hello world”, select

File>Sketchbook>Arduino-0017>Examples>Digital>Blink

Once we have our skecth we’ll see something very close to the screenshot on the following page.

Fig 4.1 A screenshot of the Arduino IDE showing the "Blink" program, a simple beginner program

4.1.3 Functions Library

[10]Arduino programs are written in C/C++, although users only need define two functions to make a runnable program:

- setup() – a function run once at the start of a program that can initialize settings

- loop() – a function called repeatedly until the board powers off

A typical first program for a microcontroller simply blinks an LED on and off. In the Arduino environment, the user might write a program like this:

#define LED_PIN 13

void setup () {

pinMode (LED_PIN, OUTPUT);     // enable pin 13 for digital output

}

void loop () {

digitalWrite (LED_PIN, HIGH);  // turn on the LED

delay (1000);                  // wait one second (1000 milliseconds)

digitalWrite (LED_PIN, LOW);   // turn off the LED

delay (1000);                  // wait one second

}

It is a feature of most Arduino boards that they have an LED and load resistor connected between pin 13 and ground, a convenient feature for many simple tests. The above code would not be seen by a standard C++ compiler as a valid program, so when the user clicks the "Upload to I/O board" button in the IDE, a copy of the code is written to a temporary file with an extra include header at the top and a very simple main() function at the bottom, to make it a valid C++ program. The Arduino IDE uses the GNU toolchain and AVR Libc to compile programs, and uses avrdude to upload programs to the board. As the Arduino platform uses Atmel microcontrollers Atmel’s development environment, AVR Studio or the newer Atmel Studio, may also be used to develop software for the Arduino.

## setup()

The setup() function is called when a sketch starts. Use it to initialize variables, pin modes, start using libraries, etc. The setup function will only run once, after each powerup or reset of the Arduino board.

### Example

int buttonPin = 3;

void setup()

{

Serial.begin(9600);

pinMode(buttonPin, INPUT);

}

void loop()

{

// ...

}

# loop()

After creating a setup() function, which initializes and sets the initial values, the loop() function does precisely what its name suggests, and loops consecutively, allowing your program to change and respond. Use it to actively control the Arduino board.

### Example

const int buttonPin = 3;

// setup initializes serial and the button pin

void setup()

{

Serial.begin(9600);

pinMode(buttonPin, INPUT);

}

// loop checks the button pin each time,

// and will send serial if it is pressed

void loop()

{

if (digitalRead(buttonPin) == HIGH)

Serial.write('H');

else

Serial.write('L');

delay(1000);

}

## Define

#define is a useful C component that allows the programmer to give a name to a constant value before the program is compiled. Defined constants in arduino don't take up any program memory space on the chip. The compiler will replace references to these constants with the defined value at compile time. This can have some unwanted side effects though, if for example, a constant name that had been #defined is included in some other constant or variable name. In that case the text would be replaced by the #defined number (or text).

In general, the const keyword is preferred for defining constants and should be used instead of #define.

Arduino defines have the same syntax as C defines:

#### Syntax

#define constantName value

Note that the # is necessary.

#### Example

#define ledPin 3

// The compiler will replace any mention of ledPin with the value 3 at compile time.

## #include

#include is used to include outside libraries in your sketch. This gives the programmer access to a large group of standard C libraries (groups of pre-made functions), and also libraries written especially for Arduino.

The main reference page for AVR C libraries (AVR is a reference to the Atmel chips on which the Arduino is based) is here.

Note that #include, similar to #define, has no semicolon terminator, and the compiler will yield cryptic error messages if you add one.

#### Example

This example includes a library that is used to put data into the program space flash instead of ram. This saves the ram space for dynamic memory needs and makes large lookup tables more practical.

#include <avr/pgmspace.h>

prog_uint16_t myConstants[] PROGMEM = {0, 21140, 702  , 9128,  0, 25764, 8456,

0,0,0,0,0,0,0,0,29810,8968,29762,29762,4500};

## pinMode()

#### Description

Configures the specified pin to behave either as an input or an output. See the description of digital pins for details on the functionality of the pins.

As of Arduino 1.0.1, it is possible to enable the internal pullup resistors with the mode INPUT_PULLUP. Additionally, the INPUT mode explicitly disables the internal pullups.

#### Syntax

pinMode(pin, mode)

#### Parameters

pin: the number of the pin whose mode you wish to set

mode: INPUT, OUTPUT, or INPUT_PULLUP. (see the digital pins page for a more complete description of the functionality.)

#### Returns

None

#### Example

int ledPin = 13;                 // LED connected to digital pin 13

void setup()
{
  pinMode(ledPin, OUTPUT);      // sets the digital pin as output
}

void loop()
{
  digitalWrite(ledPin, HIGH);   // sets the LED on
  delay(1000);                  // waits for a second
  digitalWrite(ledPin, LOW);    // sets the LED off
  delay(1000);                  // waits for a second
}

## digitalRead()

#### Description

Reads the value from a specified digital pin, either HIGH or LOW.

#### Syntax

digitalRead(pin)

#### Parameters

pin: the number of the digital pin you want to read (int)

#### Returns

HIGH or LOW

#### Example

int ledPin = 13; // LED connected to digital pin 13

int inPin = 7;   // pushbutton connected to digital pin 7

int val = 0;     // variable to store the read value

void setup()

{

pinMode(ledPin, OUTPUT);      // sets the digital pin 13 as output

pinMode(inPin, INPUT);      // sets the digital pin 7 as input

}

void loop()

{

val = digitalRead(inPin);   // read the input pin

digitalWrite(ledPin, val);    // sets the LED to the button's value

}

Sets pin 13 to the same value as the pin 7, which is an input.

## digitalWrite()

#### Description

Write a HIGH or a LOW value to a digital pin.

If the pin has been configured as an OUTPUT with pinMode(), its voltage will be set to the corresponding value: 5V (or 3.3V on 3.3V boards) for HIGH, 0V (ground) for LOW. If the pin is configured as an INPUT, writing a HIGH value with digitalWrite() will enable an internal 20K pullup resistor (see the tutorial on digital pins). Writing LOW will disable the pullup. The pullup resistor is enough to light an LED dimly, so if LEDs appear to work, but very dimly, this is a likely cause. The remedy is to set the pin to an output with the pinMode() function.

NOTE: Digital pin 13 is harder to use as a digital input than the other digital pins because it has an LED and resistor attached to it that's soldered to the board on most boards. If you enable its internal 20k pull-up resistor, it will hang at around 1.7 V instead of the expected 5V because the onboard LED and series resistor pull the voltage level down, meaning it always returns LOW. If you must use pin 13 as a digital input, use an external pull down resistor.

#### Syntax

digitalWrite(pin, value)

#### Parameters

pin: the pin number

value: HIGH or LOW

#### Returns

none

#### Example

int ledPin = 13;                 // LED connected to digital pin 13

void setup()

{

pinMode(ledPin, OUTPUT);      // sets the digital pin as output

}

void loop()

{

digitalWrite(ledPin, HIGH);   // sets the LED on

delay(1000);                  // waits for a second

digitalWrite(ledPin, LOW);    // sets the LED off

delay(1000);                  // waits for a second

}

Sets pin 13 to HIGH, makes a one-second-long delay, and sets the pin back to LOW.

## Serial

Used for communication between the Arduino board and a computer or other devices. All Arduino boards have at least one serial port (also known as a UART or USART): Serial. It communicates on digital pins 0 (RX) and 1 (TX) as well as with the computer via USB. Thus, if you use these functions, you cannot also use pins 0 and 1 for digital input or output. You can use the Arduino environment's built-in serial monitor to communicate with an Arduino board. Click the serial monitor button in the toolbar and select the same baud rate used in the call to begin(). The Arduino Mega has three additional serial ports: Serial1 on pins 19 (RX) and 18 (TX), Serial2 on pins 17 (RX) and 16 (TX), Serial3 on pins 15 (RX) and 14 (TX). To use these pins to communicate with your personal computer, you will need an additional USB-to-serial adaptor, as they are not connected to the Mega's USB-to-serial adaptor. To use them to communicate with an external TTL serial device, connect the TX pin to your device's RX pin, the RX to your device's TX pin, and the ground of your Mega to your device's ground. (Don't connect these pins directly to an RS232 serial port; they operate at +/- 12V and can damage your Arduino board.) The Arduino Due has three additional 3.3V TTL serial ports: Serial1 on pins 19 (RX) and 18 (TX); Serial2 on pins 17 (RX) and 16 (TX), Serial3 on pins 15 (RX) and 14 (TX). Pins 0 and 1 are also connected to the corresponding pins of the ATmega16U2 USB-to-TTL Serial chip, which is connected to the USB debug port. Additionally, there is a native USB-serial port on the SAM3X chip, SerialUSB'. The Arduino Leonardo board uses Serial1 to communicate via TTL (5V) serial on pins 0 (RX) and 1 (TX). Serial is reserved for USB CDC communication. For more information, refer to the Leonardo getting started page and hardware page.

## analogRead()

#### Description

Reads the value from the specified analog pin. The Arduino board contains a 6 channel (8 channels on the Mini and Nano, 16 on the Mega), 10-bit analog to digital converter. This means that it will map input voltages between 0 and 5 volts into integer values between 0 and 1023. This yields a resolution between readings of: 5 volts / 1024 units or, .0049 volts (4.9 mV) per unit. The input range and resolution can be changed using analogReference().

It takes about 100 microseconds (0.0001 s) to read an analog input, so the maximum reading rate is about 10,000 times a second.

#### Syntax

analogRead(pin)

#### Parameters

pin: the number of the analog input pin to read from (0 to 5 on most boards, 0 to 7 on the Mini and Nano, 0 to 15 on the Mega)

#### Returns

int (0 to 1023)

#### Note

If the analog input pin is not connected to anything, the value returned by analogRead() will fluctuate based on a number of factors (e.g. the values of the other analog inputs, how close your hand is to the board, etc.).

#### Example

int analogPin = 3;     // potentiometer wiper (middle terminal) connected to analog pin 3

// outside leads to ground and +5V

int val = 0;           // variable to store the value read

void setup()

{

Serial.begin(9600);          //  setup serial

}

void loop()

{

val = analogRead(analogPin);    // read the input pin

Serial.println(val);             // debug value

}

# Boolean Operators

These can be used inside the condition of an if statement.

### && (logical and)

True only if both operands are true, e.g.

if (digitalRead(2) == HIGH  && digitalRead(3) == HIGH) { // read two switches

// ...

}

is true only if both inputs are high.

## if (conditional) and ==, !=, <, > (comparison operators)

if, which is used in conjunction with a comparison operator, tests whether a certain condition has been reached, such as an input being above a certain number. The format for an if test is:

if (someVariable > 50)

{

// do something here

}

The program tests to see if someVariable is greater than 50. If it is, the program takes a particular action. Put another way, if the statement in parentheses is true, the statements inside the brackets are run. If not, the program skips over the code.

The brackets may be omitted after an if statement. If this is done, the next line (defined by the semicolon) becomes the only conditional statement.

if (x > 120) digitalWrite(LEDpin, HIGH);

if (x > 120)

digitalWrite(LEDpin, HIGH);

if (x > 120){ digitalWrite(LEDpin, HIGH); }

if (x > 120){

digitalWrite(LEDpin1, HIGH);

digitalWrite(LEDpin2, HIGH);

}                                 // all are correct

Chapter 5

List of References

- M. Thanigavel “Li-Fi Technology in Wireless Communication”, International Journal of Engineering Research & Technology, Vol. 2 Issue October 10, 2013.

- Li-Fi an Emerging Technology "Li-Fi: Future Mobile Applications by Light" Presentation on Li-Fi technology by Professor Harald Haas to Global Leaders Forum in Seoul, South Korea in November 2013.

- 4. 4 H. Haas, 'High-speed wireless networking using visible light' SPIE (The international society for optics and photonics),doi:10.1117/2.1201304.004773, April 2013.

- International journal of innovative research in electrical, instrumentation, electronics and control engineering vol.4, issue 5, May 2016.

- Prerna Chauhan, Jyoti Rani, Ritika Tripathi, “Li-Fi (Light Fidelity)-The future technology in Wireless communication”, International Journal of Applied Engineering Research, PP. 0973-4562 Vol.7, 2012. 6

- . “Li-Fi the path to a new way of communication” Information Systems and Technologies (CISTI), 12th Iberian Conference, 2017.

- 7. W.L. Jin, “SPIVC: A Smartphone-based inter-vehicle communication system,” Proceedings of Transportation Research Board Annual Meet, 2012.

- . Dobroslav Tsonev, Harald Haas and Stefan Videv, “Light Fidelity: Towards All-Optical Networking”, Institute for Digital Communications, Li-Fi R&D Centre, Edinburgh, UK.

- H. Haas, Cheng Chen “What is Li-Fi?”Optical Communication, European Conference on, PP. 1-3, 2015.